USFWS and NMFS Crooked River Act

Draft Recommendation for 2024 to 2025 / July 23, 2024

This document provides the U.S. Fish and Wildlife Service (FWS) and National Marine Fisheries Service (NMFS) draft Crooked River flow recommendation for 2024 to 2025, pursuant to the consultation requirement under Section 4 of the Crooked River Collaborative Water Security and Jobs Act of 2014 (CRA). The FWS and NMFS (collectively the Services) are submitting this draft proposed annual release schedule for the Uncontracted Storage account and the contracted City of Prineville Groundwater Mitigation account per the CRA. The CRA requires the Bureau of Reclamation (Reclamation) to consult with the Services to develop an annual release schedule that “…maximizes, to the maximum extent practicable, benefits to downstream fish and wildlife.”

In addition to the Services’ obligation under the CRA, the FWS and NMFS concluded their Deschutes Basin Habitat Conservation Plan (DBHCP) development and approval process on December 31, 2020, and October 18, 2022, respectively, and issued Incidental Take Permits, to the City of Prineville and eight Central Oregon irrigation districts (collectively the Permittees). The DBHCP was designed to work with the CRA and we have considered the applicable elements of the DBHCP while drafting our CRA recommendation.

Reclamation has determined that the Day of Allocation for Prineville Reservoir occurred on May 28, 2024.  Reclamation’s May 28, 2024 Final 2024 Storage Allocation Table is available at https://www.usbr.gov/pn/ccao/projects/PrinevilleResStorage/PrinevilleResStorage.html.  This year Prineville Reservoir filled to 148,633 acre-feet (acft) which provides the following storage quantities for release as fish and wildlife flows: 1) 60,055 acft of uncontracted storage; and

2) 4,899 acft of Prineville groundwater Mitigation Storage for a total of 64,954 acft.  These storage amounts are lower than those available during years prior to water year 2023 due to Reclamation’s accounting adjustments for reservoir storages losses from sedimentation and evaporation.  These effects are being applied to all storage accounts.  The Services believe that the following 2024 to 2025 recommendation most effectively achieves the CRA’s goal of maximizing, to the maximum extent practicable, the benefits to downstream fish and wildlife.

USFWS and NMFS 2024 to 2025 Instream Flow Recommendation

The Services consider that the following  recommendation most effectively achieves the CRA’s goal of maximizing, to the maximum extent practicable, the benefits to downstream fish and wildlife.  If implemented, our recommendation will leave approximately 20,105 acft of Uncontracted Storage as carryover for the 2025 to 2026 water year.

The Services and Reclamation are continuing our efforts to provide instream protection for fish and wildlife releases from Uncontracted Storage.  In June of 2023 the Service, Reclamation, Deschutes River Conservancy, Oregon Water Resources Department (OWRD), and others drafted an instream lease application to protect Uncontracted Storage releases from diversion as live flow.  The application was filed with the OWRD on June 14, 2023.  However, on July 14, 2023 the OWRD concluded that the instream lease approach would not be approved.  Previously on June 8, 2021, Reclamation had also filed an application for a secondary water storage right (S-89128) with the OWRD to protect Uncontracted Storage releases from diversion.  That application is still pending before the OWRD.  As a result 2024 to 2025 Uncontracted Storage releases do not have instream protection.  The Service’s Irrigation Season flow recommendation takes this lack of protection into account.  However, if the OWRD approves the application for instream flow protection the Services may revise this recommendation.  Our recommended flows are summarized in Table 1 below.

Table 1  Recommended 2024 to 2025 Instream Flows (cfs) by Storage Account

|  | June | July | Aug | Sept | Oct 15 | Nov | Dec | Jan | Feb | Mar | April |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uncontracted | 0 | 0 | 0/40 | 40 | 40/60/ 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Mitigation | 20 | 20 | 20 | 20 | 20 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | 20 | 20 | 60 | 60 | 60/60/ 100 | 100 | 100 | 100 | 100 | 100 | 100 |

- Irrigation Season Mitigation Account release / 123 days of 20 cfs flow June 10 to October 11 = 4,899 acft

- Irrigation Season Uncontracted Storage Account release / 56 days of 40 cfs flow August 16 to October 11 plus 3 days of 60 cfs flow = 3,292 acft

- Storage Season Uncontracted Storage Account release / 183 days of 100 cfs flow Oct 15 to April 14 =  36,298 acft

Irrigation Season

The Service’s recommendation for the 2024 irrigation season is described below.  During the irrigation season Crooked River instream flow will be maintained by releases from the Mitigation Storage Account and Uncontracted Storage Account.  In combination with North Unit Irrigation District’s (NUID) call for releases from their Contracted Storage Account these flow releases are intended to maintain a flow of 60 cfs at the CAPO gauge. The Services recognize that NUID storage releases may vary during the remainder of the 2024 irrigation season and that Reclamation’s guidelines for requesting flow changes may require minor adjustments to start dates for our recommended flows.  We recommend that Reclamation adjust releases from the Prineville Mitigation Storage and Uncontracted Storage accounts as needed to maintain the recommended target flows.

June 10 to October 11 Mitigation Storage Release:  Our recommendation is designed to provide a Mitigation Storage minimum flow of 20 cfs at the CAPO gauge in Prineville at RM 46.7 during the 2024 Irrigation Season from June 10 to October 11.  We recommend that this flow be released from the Mitigation Storage Account since these flows will have instream protection from diversion.  Our recommendation will exhaust the Mitigation Storage Account by October 11.  Crooked River flows at CAPO will be higher than 20 cfs when the NUID calls for irrigation storage releases from Bowman Dam at RM 70.5.  This is because NUID releases from their storage account will remain instream until diverted at the NUID’s pumps at RM 22.4.

August 16 to October 11 Uncontracted Storage Release:  The NUID estimates that they will end their call for irrigation releases on or about August 15.  After NUID’s releases end the Service’s recommend that Reclamation release an additional 40 cfs of instream flow from the Uncontracted Storage account.  Together with the continued release of 20 cfs from the Mitigation Storage Account the total instream flow at CAPO is intended to be approximately 60 cfs.

October 12 to 14 Uncontracted Storage Release:  During this period there will be no remaining Mitigation Storage Account water to release as instream flow.  We recommend that Reclamation increase the release from Uncontracted Storage by an additional 40 cfs for a total Uncontracted Storage release of 60 cfs.

Storage Season

We recommend that 100 cfs of fish and wildlife flow be released from Uncontracted Storage Account from October 15, 2024, to April 14, 2025.  Our recommendation will use about 36,298 acft of Uncontracted Storage.

Smolt Outmigration Flow and/or Flow Enhancement:  The Services will determine what smolt outmigration or supplemental instream flows will maximize the benefits to steelhead, bull trout, spring Chinook, and other native fish species in the Crooked River after consulting and coordinating with the Confederated Tribes of the Warm Springs (CTWS), the Oregon Department of Fish and Wildlife (ODFW), and the Pelton Round Butte Project Fish Committee.  Flows could either be supplemented to increase Storage Season flows and/or provide water to perform ‘pulse flows’ to assist spring Chinook and steelhead smolts migrating out of the Crooked River system in the Spring of 2025. It may also be possible to release additional flow during the spring chinook migration and spawning season.  The specific details of these releases will consider the best available real-time information on river conditions, smolt out-planting dates, and other habitat and climate conditions at the time. The Services and our partners will seek to educate the public regarding these flow changes using existing public outreach strategies.

Reclamation Flood Control Operations:  If Reclamation conducts flood control operations during the 2024 to 2025 period addressed by this recommendation the Service’s request that Reclamation cease all releases from Uncontracted Storage until flood control operations are concluded. This is because flood control releases will already be meeting the Service’s recommended 100 cfs instream flow for fish and wildlife.  It is our understanding that there can be intermittent periods of flood control followed by a return to normal storage season operations.  During these periods of normal storage season operations we recommend that Reclamation release 100 cfs from Uncontracted Storage.

City of Prineville Groundwater Mitigation Storage Account

We recommend that the 4,899 acft of Mitigation Storage be released to provide an additional 20 cfs of flow from June 10 to October 11.  Details regarding these releases are provided above.  We also recommend that Reclamation request that the OWRD ensure that flows released from the Mitigation Storage acft account are fully protected as instream flow after they are released from Prineville Reservoir.  It is our understanding that Reclamation as the party holding the secondary water right for the Mitigation Storage account must request this protection every year.

Rationale for USFWS/NMFS 2024 to 2025 Recommendation

Our recommendation is based on the following rationale.

- Our recommendation takes into consideration that releases of fish and wildlife flows from Uncontracted Storage are considered “live flow” under Oregon state law.  This is because Uncontracted Storage releases do not have the protection of a secondary water right.  As a result, they may be diverted by parties downstream of Bowman Dam that may have unmet live flow diversion rights.  Since these flows are vulnerable to diversion, particularly during the Irrigation Season, their benefits to fish and wildlife are maximized by retaining them for Storage Season release, or as carryover for the next water year.  Alternatively, Irrigation Season flows can be maintained by releases from the City of Prineville’s Groundwater Mitigation account since releases from this account have instream protection.  As previously noted, efforts to provide instream protection to releases from Uncontracted Storage have so far been unsuccessful.

- Our recommendation will allow Storage Season flows to be maintained at higher levels.  During the winter Storage Season flows from Prineville Reservoir are needed to support fish species from Bowman Dam at river mile 70.5 to the river's confluence with Lake Billy Chinook, since at this time of the water year there is no irrigation flow to augment fish and wildlife instream flows.  During the storage season the approximately 26 miles of the Crooked River from Bowman Dam to the confluences of Ochoco and Mckay Creeks are almost entirely dependent on Prineville Reservoir fish and wildlife flows.

- Our recommendation will provide about 20,105 acft of Uncontracted Storage carryover for the 2025 to 2026 water year.

- Our recommendation will provide chinook, steelhead, redband rainbow trout habitat during the Irrigation Season and Storage Season.

Irrigation Season:  Our recommendation to release 20 cfs of flow from the Groundwater Mitigation account from June 10 to October 11 together with a supplementary release of 40 cfs from Uncontracted Storage after NUID ends its call for irrigation storage releases on or about August 15 is intended maintain a flow of about 60 cfs through the Crooked’s low flow reach.  This reach extends about 6.4 miles from People’s Diversion at about RM 49.4, past the CAPO gauge at RM 46.7, and down to the confluences of Ochoco and Mckay Creeks between RM 43.9 to RM 43.  After NUID’s calls for irrigation season releases end in the Fall we recommend that 60 of instream flow be released from Uncontracted Storage.

The Hardin IFIM study (Hardin, 1993) focused on redband rainbow trout habitat, and in 2001 Hardin used the 1993 data to generate IFIM habitat quantities for Chinook and steelhead (Hardin, 2001).  The IFIM study for the Crooked River divided the Crooked River into three reaches which are: 1) Upper Canyon Reach from Bowman Dam RM 70.5 to Stearns Diversion Site at RM 59; 2) Valley Reach from Stearns Diversion RM 59 to the NUID Pumps RM 22.4; and 3) Lower Canyon reach from NUID pumps RM 22.4 to the Highway 97 bridge RM 12.4.  The percent of total habitat available for life stages of Chinook, steelhead, and redband rainbow trout are shown below in Table 2.

Table 2  60 cfs Flow Steelhead, Chinook, and Redband Rainbow Trout Habitat

|  | Upper Canyon % | Valley % | Lower Canyon % |
| --- | --- | --- | --- |
| Juvenile Steelhead | 81.5 | 80.1 | 83.1 |
| Spawning Steelhead | 29.8 | 86.8 | 21.7 |
| Juvenile Chinook | 81.9 | 80.1 | 90.2 |
| Spawning Chinook | 12.3 | 71.9 | 17.8 |
| Redband Fry | 47.0 | 27.8 | 42.4 |
| Redband Juvenile | 58.9 | 54.9 | 61.7 |
| Redband Adult | 13.2 | 18.9 | 15.0 |
| Redband Spawn | 0.4 | 0.9 | 0.9 |

Storage Season:  From October 15, 2024, to April 14, 2025, we recommend releasing 100 cfs of flow from the Uncontracted Storage account.  This will maintain minimum instream flows from Bowman Dam at RM 70.5 to the confluences of Ochoco and Mckay Creeks between RM 43.9 to RM 43.  Inflow from these creeks will further increase instream flows.  The percent of total habitat available for life stages of Chinook, steelhead, and redband rainbow trout are shown below in Table 3.

Table 3  100 cfs Flow Steelhead, Chinook, and Redband Rainbow Trout Habitat

|  | Upper Canyon % | Valley % | Lower Canyon % |
| --- | --- | --- | --- |
| Juvenile Steelhead | 94.0 | 95.1 | 96.7 |
| Spawning Steelhead | 74.9 | 92.5 | 71.2 |
| Juvenile Chinook | 97.4 | 95.6 | 99.1 |
| Spawning Chinook | 49.8 | 99.2 | 57.4 |
| Redband Fry | 45.1 | 26.6 | 40.5 |
| Redband Juvenile | 62.4 | 58.6 | 66.7 |
| Redband Adult | 20.7 | 28.5 | 22.2 |
| Redband Spawning | 0.5 | 0.8 | 1.4 |

USFWS and NMFS Coordination with Oregon Fisheries Co-Managers

The Service’s sent our draft recommendation to the ODFW and CTWS on Thursday, June 27 and requested comments by Friday, July 5. The ODFW replied with comments on July 5.  The Services replied to those comments on July 10 and has updated elements of our draft recommendation.  The CTWS has not provided any comments on the draft.

Conclusion

The Services believe that this 2024 to 2025 CRA recommendation most effectively achieves the Crooked River legislation's goal of maximizing, to the maximum extent practicable, the benefits to downstream fish and wildlife.  Please let us know if you have any questions or would like additional information.

Citations

Hardin, T.  1993.  Summary Report Crooked River Instream Flow Study.  Prepared for the Oregon Department of Fish and Wildlife, U.S. Bureau of Land Management, and U.S. Bureau of Reclamation.

Hardin, T.  2001.   Physical Habitat for Anadromous Species in the Crooked River Below Bowman Dam.  Prepared for the Pacific Northwest Region U.S. Bureau of Reclamation.

https://www.fws.gov/project/deschutes-river-basin-habitat-conservation-plan

https://www.fisheries.noaa.gov/west-coast/habitat-conservation/habitat-conservation-plans-west-coast